Water loss from intermittently closed estuaries owing to berm seepage

: Russian River case study

Methods …
… note that travel time through berm expected to be many hours (sand K ~ 400m/day and berm width ~ 100m), so expect smoothing out of tidal and diurnal and other high-frequency fluctuations; allows more convenient approach of using daily values.

- I have made note of this in the methods

… if linear relation between S and dh it’s okay to use daily average h, but if very non-linear then we should use hourly h and average hourly S to get daily value.

- I’m not sure I see the connection between the linearity of the relation between S and dh and hourly vs. daily averaging.

… interpret loss as seepage: (i) seepage values well behaved increasingly smoothly, (ii) seepage scales well with increasing head difference, and (ii) other terms are order of magnitude smaller, like evaporation.

- How would you like to add this comment to the methods?

… estimate E using 100 ha

I have corrected this.

… could manually identify all overwash days by reviewing photos during selected long closures; otherwise acknowledge that there are still some days that haven’t been removed.

- We could, but photos are not available for some of the large closures.  I did manually review quite a few photos to check

… write Eqn 3 for S not Qb

I’ve fixed this.

… for Eqn 4 we could write it as an integral over dy and dz, then simplify to form you give and end with writing S = K~.dh

Why do we want to do this?

… we are using expanded area for higher WL, right?  So WL increase slows down just because of hypsometry

I’m not sure what you mean by expanded area.

Results …

… plot of S vs dh for several examples, each with zero-intercept linear and non-linear best-fit (y = p +q.(dh)r

- Let’s talk about this one.

… table of results for 30 events longer than 5 days with overwash days removed: date, # days, linear slope, linear r2, non-linear power, non-linear r2

- Agreed, I’ll add this table.  Let’s talk about the non-linear power – I’m not sure I agree

… plot of S vs dh for all data with a single result (linear and non-linear?) and envelope lines (K~ = 0.5 and 1.5 or whatever we choose)

- You want to pool all of the data into a single estimate of K~?  Why?

… remove old Figure 5 (perhaps I am missing the key point related to this)

- No point, really. It shows that the seepage rate tends to increase over the duration of the closure.

… in Fig 4 maybe drop panel (c) and only show orange dots in panel (b); also let’s join the dots and demarcate years when berm resets.

- Let’s talk about this.   I think (c) is interesting from a mass balance perspective.  A list of month/years when the berm resets would be useful.

… final paragraph seems unnecessary in Results

- This is related to Figure 5.  Within-closure trends in seepage.

Discussion

Water loss from the Russian River Estuary via seepage through the sand barrier is well behaved (e.g., Figure 3), increasing as expected with an increase in head, i.e., the difference between estuary water level and ocean water level. Observed seepage losses are of order 1 m3/s and can increase to more than 2 m3/s under the current management regime under which the mouth is breached when estuary head exceeds about 2m (9ft above MMLW). The integrated hydraulic conductivity K~ represents the hydraulic conductivity K with the cross-sectional area of the flow (product of width W and depth of flow H) and the length of the flow path L along which hydraulic pressure drops from estuary to ocean. As the estuary water level rises, the hydraulic gradient increases and one may expect the depth of flow to also increase, yielding a quadratic dependence on estuary water level (and if width of flow increases or length of flow path decreases with increasing water level, one may expect even stronger dependence on estuary water level).  However, when fitting seepage rate S to estuarine head dh, the best fit occurs for an exponent of 1.25 and the fit is not much better than a simpler linear fit, which we use because it gives us single parameter as an index of seepage response to estuarine head. However, the low value of the exponent indicates that neither the depth nor width of the flow changes significantly with increasing estuarine water level, which in turn suggests a preferred conduit for seepage (without any change in cross-sectional area). Further, a back-of-the envelope estimate of seepage through a coarse sand barrier with K ~ 0.00052 m/s, W ~ 300m, H ~ 5m, and L ~ 50m yields a flux of ~0.02 m3/s for every 1 m of estuarine head. For a gravel berm, the seepage flux increases to ~0.09 m3/s. This suggests that the barrier is not uniform and rather includes some high-conductivity conduits, which can “pipe” the water through the and barrier, perhaps associated with the remnant jetty or naturally occurring beneath the coarse sand berm. More realistically, the integrated hydraulic conductivity is an integration of non-uniform K(y,z) over vertical z and cross-stream y dimensions of the flow through the barrier.

While K~ has an aggregate value close to 1.0 m2/s, it varies between closure events and displays an interesting tendency for decrease from higher values in early summer to lower values in the fall (e.g., K~ values in 2017, Figure 4). The lowest values ~0.5 m2/s are found after winters in which the barrier was not removed and rebuilt (e.g., K~ decreases from 2019 to 2020 to 2021) and the highest values ~1.5 m2/s are found after a major winter flow event (e.g., early 2017). Seasonal and interannual reduction in integrated hydraulic conductivity K~ with time suggests that the aquifer becomes clogged, but it could also result from an increase in the length of the flow path as the barrier thickens, aggrading seaward during calm periods.

Uncertainty in seepage estimates could be due to several factors, but the smooth variation in S values from day to day (in the absence of wave overwash) suggests that uncertainty is limited, accounting for variability of order 0.1 m3/s, which is an order of magnitude smaller than seepage rates. Errors in the mass balance calculation can come from (i) neglecting evaporative loss of freshwater, (ii) unaccounted wave overwash events, (iii) groundwater exchange, (iv) differences in ocean water level between Point Reyes gage at Russian River mouth associated with wind and wave setup, and (v) unaccounted creek inflow. We have estimated contributions from evaporation and inflow from Austin Creek (major tributary) and find them both negligible. Also, we have looked for high-frequency variability in S estimates associated with error in ocean level estimates due to wind or wave setup and find no high-frequency variability in S values during short-lived wind or wave events. While deSieyes et al (2010?) quantify tidal pumping from the beach at a site in Stinson Beach, in the case of closed-mouth estuaries, the flux through the sand is dominated by the quasi-steady head due to a perched estuary water level.

Clearly the estuary exports water through the sand barrier during closure, and this loss grows with increased elevation of the water level in the estuary. Ultimately, if the berm is high enough, seepage will increase to match the rate of river inflow and the estuary water level will remain steady. This steady state is never achieved in the Russian River Estuary as the mouth is breached mechanically before estuarine head reaches 2 m, when seepage can account for a loss of about 2 m3/s (approximately 60 cfs), which is less than typical summer river flows that are managed to maintain adequate water availability in the lower river. However, in the 2014 drought year when river flow rate was decreased to ~ 50 cfs, seepage balanced that inflow and water level remained constant in the estuary for the first 10 days of October. In years with higher river flow (typically 100 cfs), a steady state could be achieved if water level rose to about 3 m above ocean water level. It seems likely that this occurred prior to development of the estuary and closures that started in spring could persist for months through the summer low-flow months. Alternatively, closure may have persisted with estuarine water level cresting the crest of the berm but with such a low flow rate that the berm was not scoured. In this perched estuary state one can imagine order 0.5 m3/s flowing over the sand barrier as a 10 m wide and 0.1m deep flow with velocities below 0.5 m/s (too slow to scour sand).

Prolonged closure allows a freshwater layer (epilimnion) to develop in the estuary, which offers critical habitat for freshwater-acclimated young steelhead. Further a cool, moderate salinity metalimnion also can develop, offering habitat for smolts to grow prior to leaving the estuary and entering the ocean in the fall. The deep and cold hypolimnion goes anoxic beneath extreme stratification that precludes reoxygenation and without any significant photosynthetically active radiation.

As in the Russian River Estuary, it is expected that seepage losses are significant in other ICEs where the water level is allowed to rise significantly above the ocean water level – which can only happen where river inflow continues through closure periods.  Different barrier configurations in terms of dimensions and sediment types result in different flow rates.  And may see interesting differences in K~ between seasons and years due to clogging and/or changes in berm shape. More work remains to be done.  Also note reverse seepage in SoCal.

I think this is the essential content, but I ran out of steam at the end and know that I could write it better (and with some citation of other work!).